COMPARATIVE REPORT  •  CPU SCHEDULING

LIFO versus

Rate Monotonic and

Earliest Deadline First

A theoretical and experimental study of a non-real-time stack-based policy against the two classic real-time scheduling algorithms, using six periodic task sets and a Python simulator.

| Field | Details |
| --- | --- |
| Prepared by | FATEMEH |
| Student ID |  |
| Course | Operating Systems |
| Instructor |  |
| Date | October 2026 |

Contents

1. Introduction	4

1.1 Background	4

1.2 Objectives	4

1.3 Structure of the report	4

2. Fundamental Concepts	5

2.1 Classification of scheduling algorithms	5

2.2 The task model used	6

3. LIFO Scheduling	7

3.1 Definition	7

3.2 Variants	7

3.3 Algorithm and pseudo-code	7

3.4 A small example	8

3.5 Where LIFO is used in practice	8

3.6 Advantages and disadvantages	8

4. Rate Monotonic Scheduling	9

4.1 Definition	9

4.2 Pseudo-code	9

4.3 Optimality	9

4.4 The Liu & Layland utilization bound	9

4.5 The hyperbolic bound	10

4.6 Response Time Analysis (exact test)	10

4.7 Harmonic task sets	10

4.8 Advantages and disadvantages	10

5. Earliest Deadline First	12

5.1 Definition	12

5.2 Pseudo-code	12

5.3 Optimality and the utilization test	12

5.4 Processor demand analysis	12

5.5 Behaviour under overload: the domino effect	12

5.6 Advantages and disadvantages	13

6. Theoretical Comparison	14

7. Experimental Methodology	16

7.1 The simulator	16

7.2 Metrics	16

7.3 Scenarios	17

8. Step-by-Step Worked Example	18

8.1 LIFO-NP	18

8.2 LIFO-P	18

8.3 RM	18

8.4 EDF	19

8.5 What the traces show	19

9. Experimental Results	20

9.1 Scenario S1: light load (U = 0.583)	20

9.2 Scenario S2: above the RM bound (U = 0.833)	21

9.3 Scenario S3: high load (U = 0.950)	23

9.4 Scenario S4: harmonic set at full load (U = 1.000)	24

9.5 Scenario S5: five tasks (U = 0.800)	26

9.6 Scenario S6: overload (U = 1.250)	27

10. Summary Across Scenarios	30

11. Discussion	33

11.1 Why LIFO fails as a real-time scheduler	33

11.2 Preemptive versus non-preemptive LIFO	33

11.3 Misleading metrics	33

11.4 RM versus EDF	33

11.5 Threats to validity	33

12. Practical Recommendations	34

13. Conclusion	35

References	36

Appendix A. Simulator Source Code	37

Appendix B. Raw Program Output	41

# 1. Introduction

## 1.1 Background

Every multitasking system must repeatedly answer the same question: which of the tasks that are ready to run should receive the processor next? The rule that answers this question is the scheduling algorithm. The right rule depends on what the system is trying to achieve. A desktop operating system cares about throughput, fairness and the responsiveness felt by the user, while a real-time system, such as the controller of an airbag, a pacemaker or an industrial robot, cares above all about predictability: every task must finish before its deadline, every time.

This report studies three algorithms that come from these two different worlds. LIFO (Last-In, First-Out) is a simple ordering policy borrowed from the stack data structure: the task that arrived most recently is served first. It has no notion of time limits. Rate Monotonic (RM) and Earliest Deadline First (EDF) are the two most important algorithms in real-time scheduling theory and were both analysed in the classic 1973 paper by Liu and Layland. RM gives fixed priorities according to task rates, while EDF gives dynamic priorities according to absolute deadlines.

## 1.2 Objectives

- Explain how LIFO, RM and EDF work, including their variants, data structures and pseudo-code.

- Present the analytical schedulability tests that exist for RM and EDF and explain why no such test exists for LIFO.

- Implement all algorithms in one Python simulator and run them on the same six periodic task sets, from light load to overload.

- Compare the algorithms quantitatively (deadline misses, response time, waiting time, preemptions, CPU utilization) and qualitatively (predictability, overhead, fairness, overload behaviour).

- Give practical recommendations about when each algorithm is appropriate.

## 1.3 Structure of the report

Section 2 introduces the vocabulary used throughout the report. Sections 3, 4 and 5 describe LIFO, RM and EDF in detail. Section 6 compares them from a theoretical point of view. Section 7 describes the simulator and the experimental method, and Section 8 walks through a complete example step by step. Section 9 presents the results of the six scenarios, Section 10 summarises them across scenarios, and Section 11 discusses the findings. Section 12 gives recommendations and Section 13 concludes. The full source code and raw output are included in the appendices.

# 2. Fundamental Concepts

Real-time scheduling uses a precise vocabulary. The terms below are used in every later section, so they are defined here first.

TABLE 1  Terminology used in this report

| Term | Definition |
| --- | --- |
| Task (τᵢ) | A recurring activity, for example "read the sensor every 10 ms". A periodic task generates an infinite sequence of jobs. |
| Job (τᵢ.k) | One instance of a task. In this report T1.3 means the third job of task T1. |
| Release time (r) | The moment a job becomes ready to run. For periodic tasks, job k is released at (k − 1)·Tᵢ. |
| Execution time (Cᵢ) | The worst-case execution time (WCET) of each job of the task. |
| Period (Tᵢ) | The fixed time between two consecutive releases of the task. |
| Relative deadline (Dᵢ) | The time after release by which the job must finish. Here Dᵢ = Tᵢ (implicit deadlines). |
| Absolute deadline (d) | Release time plus relative deadline: d = r + Dᵢ. |
| Utilization (U) | The fraction of processor time a task set needs: U = Σ Cᵢ / Tᵢ. |
| Hyperperiod (H) | The least common multiple of all periods. The schedule repeats after H. |
| Response time (R) | Finish time minus release time of a job. |
| Waiting time (W) | Time from release until the job first gets the CPU. |
| Preemption | Taking the CPU away from an unfinished job to give it to another job. |
| Context switch | Any change of the running job; it has a cost in saving and restoring state. |
| Deadline miss | A job is not finished by its absolute deadline. |
| Schedulable | An algorithm can meet all deadlines of the task set. |
| Feasible | Some algorithm exists that can meet all deadlines of the task set. |
| Optimal algorithm | An algorithm that schedules every task set that is feasible within its class. |
| Starvation | A ready job waits indefinitely because others are always preferred. |

## 2.1 Classification of scheduling algorithms

Scheduling algorithms can be classified along several independent dimensions. The table below places the three algorithms of this study in that classification.

TABLE 2  Classification of the three algorithms

| Dimension | LIFO | RM | EDF |
| --- | --- | --- | --- |
| Design goal | Simplicity, recency | Predictable real-time | Optimal real-time |
| Priority type | Dynamic (by arrival order) | Fixed per task | Dynamic per job |
| Priority basis | Arrival time (latest first) | Period (shortest first) | Absolute deadline (earliest first) |
| Preemptive | Either variant | Yes | Yes |
| Online / offline | Online | Priorities offline, dispatch online | Online |
| Uses timing parameters | No | Period only | Deadline |
| Analytical guarantee | None | Yes | Yes |

## 2.2 The task model used

All analysis and experiments use the classic periodic task model of Liu and Layland: tasks are independent (no shared resources or precedence constraints), each task is released periodically starting at time 0, relative deadlines equal periods, there is a single processor, and the cost of context switches is ignored. Time is discrete and advances in unit steps. These assumptions are standard in textbooks and make the results easy to verify by hand.

WHY USE PERIODIC TASKS FOR LIFO? LIFO is usually discussed with one-shot processes, while RM and EDF are defined for periodic tasks. To compare them fairly, all three must schedule the same workload. LIFO is therefore applied to the stream of periodic jobs: whenever a choice is needed, it picks the job that was released most recently.

# 3. LIFO Scheduling

## 3.1 Definition

Last-In, First-Out scheduling keeps the ready jobs in a stack. A newly released job is pushed on top of the stack, and the scheduler always serves the job on top, which is the one that arrived last. The policy is the mirror image of FIFO (First-Come, First-Served), which serves the oldest job first.

Because several periodic jobs can be released at exactly the same instant, a tie-breaking rule is needed. In this study, jobs released at the same time are pushed in task order (T1, then T2, then T3...), so the task with the highest index ends up on top of the stack and is considered the latest arrival. This rule is arbitrary, which is itself an important property of LIFO: the outcome depends on accidental details of arrival order rather than on the needs of the tasks.

## 3.2 Variants

TABLE 3  The two LIFO variants studied

| Property | Non-preemptive LIFO (LIFO-NP) | Preemptive LIFO (LIFO-P) |
| --- | --- | --- |
| When is a decision made? | Only when the CPU becomes free | At every release and completion |
| Effect of a new arrival | Pushed on the stack; waits for the running job | Immediately takes the CPU from the running job |
| Preemptions | Never | On every release while busy |
| Risk | Long jobs block urgent jobs | Old jobs are pushed deeper and may starve |
| Typical use | Batch processing of a stack of requests | Interrupt nesting, newest-first services |

## 3.3 Algorithm and pseudo-code

on release(job): stack.push(job) if PREEMPTIVE and running is not None: stack.push(running) # running job goes back under the new job running = None on cpu_free(): if stack is not empty: running = stack.pop() # the most recent job every time unit: drop every job whose deadline has passed # counted as a miss run running for one unit

With a linked-list stack, both push and pop take O(1) time, so LIFO has the lowest possible dispatching overhead. In the simulator, the same rule is expressed as "select the ready job with the largest release time", which gives identical decisions.

## 3.4 A small example

Consider three one-shot jobs A (arrives at 0, needs 4 units), B (arrives at 1, needs 2) and C (arrives at 2, needs 1). The table below compares FIFO with the two LIFO variants.

TABLE 4  LIFO and FIFO on three one-shot jobs

| Policy | Execution order | Finish times (A, B, C) | Average response |
| --- | --- | --- | --- |
| FIFO | A A A A B B C | 4, 6, 7 | (4 + 5 + 5) / 3 = 4.67 |
| LIFO-NP | A A A A C B B | 4, 7, 5 | (4 + 6 + 3) / 3 = 4.33 |
| LIFO-P | A B C B A A A | 7, 4, 3 | (7 + 3 + 1) / 3 = 3.67 |

Preemptive LIFO gives the shortest average response here because the short jobs B and C arrive later and are served immediately, but the first job A suffers: its response time grows from 4 to 7. If more jobs kept arriving, A could wait forever. This is the central trade-off of LIFO: it favours newcomers at the expense of older work.

## 3.5 Where LIFO is used in practice

- Interrupt handling. Nested interrupts behave in a LIFO way: a newer, higher-level interrupt suspends the current handler, which resumes only after the newer one returns.

- Work-stealing schedulers. Runtimes such as Cilk and Java’s ForkJoinPool let each worker take its own newest task first (LIFO) for cache locality, while idle workers steal the oldest tasks (FIFO) from others.

- Function calls and recursion. The call stack itself is a LIFO structure.

- Request handling under overload. Some services serve the newest requests first, because old requests have often already been abandoned by the client.

None of these uses involves hard deadlines. LIFO is chosen in them for locality, simplicity or freshness, not for timing guarantees.

## 3.6 Advantages and disadvantages

TABLE 5  Strengths and weaknesses of LIFO

| Advantages | Disadvantages |
| --- | --- |
| Extremely simple to implement (a stack) | Completely ignores deadlines and periods |
| O(1) scheduling decisions, minimal overhead | No analytical schedulability test exists |
| Good cache locality: the newest data is still warm | Old jobs can starve indefinitely |
| Short newly arrived jobs get fast response (LIFO-P) | Results depend on arbitrary tie-breaking |
| No parameters to configure | Unfair: the first arrivals are penalised |

# 4. Rate Monotonic Scheduling

## 4.1 Definition

Rate Monotonic (RM) assigns each periodic task a fixed priority that is inversely proportional to its period: the task with the shortest period (the highest rate) receives the highest priority. At run time, the scheduler always runs the ready job of the highest-priority task and preempts a lower-priority job as soon as a higher-priority job is released.

Because priorities are assigned once, before the system starts, RM is called a fixed-priority or static-priority algorithm. It is supported directly by almost every real-time operating system, since these kernels already provide preemptive fixed-priority scheduling; the designer only has to assign priorities in rate order.

## 4.2 Pseudo-code

offline: sort tasks by period, shortest first priority[task] = position in this order # 1 = highest online, at every release or completion: job = ready job with the highest priority[job.task] if job != running: preempt running and dispatch job

## 4.3 Optimality

Liu and Layland proved that RM is optimal among fixed-priority algorithms for implicit-deadline periodic tasks: if a task set can be scheduled by any fixed-priority assignment, it can also be scheduled by RM. RM is not optimal among all algorithms, however; there are task sets that EDF can schedule but RM cannot, as Scenario S3 in this report shows.

## 4.4 The Liu & Layland utilization bound

A set of n tasks is guaranteed to be schedulable by RM if U ≤ n(2^(1/n) − 1). The bound decreases as n grows and approaches ln 2 ≈ 0.693. The test is sufficient but not necessary: a task set above the bound may still be schedulable, and the more accurate tests below must then be used.

TABLE 6  Liu & Layland bound for different numbers of tasks

| n (tasks) | RM bound | Guaranteed idle capacity | EDF bound |
| --- | --- | --- | --- |
| 1 | 1.000 | 0.0 % | 1.000 |
| 2 | 0.828 | 17.2 % | 1.000 |
| 3 | 0.780 | 22.0 % | 1.000 |
| 4 | 0.757 | 24.3 % | 1.000 |
| 5 | 0.743 | 25.7 % | 1.000 |
| 6 | 0.735 | 26.5 % | 1.000 |
| 8 | 0.724 | 27.6 % | 1.000 |
| 10 | 0.718 | 28.2 % | 1.000 |
| ∞ | 0.693 | 30.7 % | 1.000 |

## 4.5 The hyperbolic bound

Bini, Buttazzo and Buttazzo (2003) proposed a tighter sufficient test: the task set is schedulable by RM if ∏(Uᵢ + 1) ≤ 2. It accepts every set the Liu & Layland test accepts, and some additional ones. In this study it is reported for every scenario.

## 4.6 Response Time Analysis (exact test)

The exact test for fixed-priority scheduling is Response Time Analysis (RTA), introduced by Joseph and Pandya (1986). The worst-case response time of task i is the smallest fixed point of the recurrence below, where hp(i) is the set of tasks with higher priority than i. The task set is schedulable if and only if Rᵢ ≤ Dᵢ for every task.

Rᵢ(0) = Cᵢ Rᵢ(k+1) = Cᵢ + Σ over j in hp(i) of ⌈Rᵢ(k) / Tⱼ⌉ · Cⱼ stop when Rᵢ(k+1) = Rᵢ(k) (or when it exceeds Dᵢ)

Example for Scenario S3 (T1 = (2, 5), T2 = (2, 8), T3 = (3, 10)), task T3:

TABLE 7  RTA iterations for task T3 of Scenario S3

| Iteration | Computation | R₃ | Status |
| --- | --- | --- | --- |
| 0 | C₃ | 3 | start |
| 1 | 3 + ⌈3/5⌉·2 + ⌈3/8⌉·2 = 3 + 2 + 2 | 7 | continue |
| 2 | 3 + ⌈7/5⌉·2 + ⌈7/8⌉·2 = 3 + 4 + 2 | 9 | continue |
| 3 | 3 + ⌈9/5⌉·2 + ⌈9/8⌉·2 = 3 + 4 + 4 | 11 | 11 > D₃ = 10: miss |

The analysis predicts that the first job of T3 can miss its deadline at time 10, which is exactly what the simulation observes (Section 9.3).

## 4.7 Harmonic task sets

When every period divides every longer period (for example 4, 8 and 16), the task set is called harmonic. For harmonic sets RM can use the processor up to 100 %, exactly like EDF. Scenario S4 demonstrates this case.

## 4.8 Advantages and disadvantages

TABLE 8  Strengths and weaknesses of RM

| Advantages | Disadvantages |
| --- | --- |
| Priorities are fixed: simple to implement on any RTOS | May leave up to about 30 % of the CPU unusable |
| Highly predictable and easy to certify | Ignores actual deadlines; only the period matters |
| Exact analysis available (RTA) | Important tasks with long periods get low priority |
| Graceful overload: only low-priority tasks miss | More preemptions than EDF in many task sets |
| Optimal among fixed-priority algorithms | Not optimal in general |

# 5. Earliest Deadline First

## 5.1 Definition

Earliest Deadline First (EDF) is a dynamic-priority algorithm. At every scheduling point, the ready job with the earliest absolute deadline gets the processor. A job of a given task can therefore have a high priority at one moment and a low priority at another, depending on how close its deadline is compared with the other ready jobs.

## 5.2 Pseudo-code

ready = min-heap ordered by absolute deadline on release(job): job.deadline = job.release + task.period ready.push(job) if job.deadline < running.deadline: preempt running and dispatch job on completion: running = ready.pop_min()

## 5.3 Optimality and the utilization test

Dertouzos (1974) showed that EDF is optimal on a single processor: if any algorithm can meet all deadlines of a set of independent jobs, EDF can as well. For implicit-deadline periodic tasks, Liu and Layland proved the simple and exact test: the task set is schedulable by EDF if and only if U ≤ 1. EDF can therefore use the whole processor, with no capacity wasted.

## 5.4 Processor demand analysis

For more general models (for example deadlines shorter than periods), EDF is analysed with the demand bound function dbf(L), the total execution time of all jobs that are both released and due inside an interval of length L. The set is schedulable if dbf(L) ≤ L for every L. For implicit deadlines the test reduces to U ≤ 1, but the table below shows the check for Scenario S2 to illustrate the idea.

TABLE 9  Processor demand check for Scenario S2

| Interval length L | Demand dbf(L) = Σ ⌊L/Tᵢ⌋·Cᵢ | dbf(L) ≤ L ? |
| --- | --- | --- |
| 4 | 1 | Yes |
| 6 | 3 | Yes |
| 8 | 4 | Yes |
| 12 | 10 | Yes |

## 5.5 Behaviour under overload: the domino effect

EDF’s weakness appears when U > 1. Because priorities follow deadlines rather than importance, a job that is already late still has the earliest deadline and keeps the processor, causing the next job to be late as well. Misses can cascade across many tasks, which is known as the domino effect. Practical EDF systems therefore add admission control or budget enforcement; for example, Linux’s SCHED_DEADLINE class combines EDF with the Constant Bandwidth Server so that one task cannot overrun its reserved share.

## 5.6 Advantages and disadvantages

TABLE 10  Strengths and weaknesses of EDF

| Advantages | Disadvantages |
| --- | --- |
| Optimal: uses up to 100 % of the CPU | Priorities change at run time: more complex kernel support |
| Simple exact test (U ≤ 1) | Unpredictable under overload (domino effect) |
| Usually fewer preemptions than RM | Requires a priority queue: O(log n) per operation |
| Directly expresses timing requirements | Less widely supported by commercial RTOSs |
| Long-period tasks are treated fairly | Harder to reason about which task will miss |

# 6. Theoretical Comparison

This section compares the three algorithms before any experiment is run, using only their definitions and the known theoretical results.

TABLE 11  Schedulability guarantees

| Question | LIFO | RM | EDF |
| --- | --- | --- | --- |
| Is there a test? | No | Yes (L&L, hyperbolic, RTA) | Yes (U ≤ 1, dbf) |
| Test type | - | Sufficient (L&L) and exact (RTA) | Exact |
| Maximum guaranteed U | 0 (nothing guaranteed) | 69.3 % to 100 % | 100 % |
| Optimal? | No | Among fixed priorities | Yes (single CPU) |
| Harmonic sets | No benefit | 100 % usable | 100 % usable |

TABLE 12  Computational complexity of a scheduling decision (n ready jobs)

| Operation | LIFO | RM | EDF |
| --- | --- | --- | --- |
| Data structure | Stack | Priority array / bitmap | Min-heap by deadline |
| Insert a released job | O(1) | O(1) with bitmap | O(log n) |
| Select next job | O(1) | O(1) with bitmap | O(1) peek, O(log n) remove |
| Offline work | None | Sort tasks by period | None |
| Parameters needed | None | Periods | Deadlines |

TABLE 13  Qualitative comparison

| Criterion | LIFO | RM | EDF |
| --- | --- | --- | --- |
| Predictability | Low | High | Medium |
| Deadline awareness | None | Indirect | Direct |
| Fairness | Poor (old jobs wait) | Biased to short periods | Good |
| Starvation | Yes | Only for low priority under overload | No when U ≤ 1 |
| Preemptions | NP: none; P: many | Medium to high | Usually lowest |
| Overload behaviour | Arbitrary victims | Low-priority tasks miss | Domino effect |
| Implementation effort | Trivial | Easy | Moderate |
| Industry support | Not used for real time | Almost every RTOS | Linux SCHED_DEADLINE, research RTOSs |

TABLE 14  Hypotheses tested by the experiments

| # | Hypothesis | Tested in |
| --- | --- | --- |
| H1 | All algorithms meet all deadlines when the load is light. | S1 |
| H2 | Above the Liu & Layland bound, RM may still succeed, but LIFO already fails. | S2, S5 |
| H3 | Between the RM limit and U = 1, only EDF meets all deadlines. | S3 |
| H4 | For harmonic sets RM reaches U = 1, like EDF. | S4 |
| H5 | Under overload all algorithms miss, but they choose different victims. | S6 |

# 7. Experimental Methodology

## 7.1 The simulator

All four policies (LIFO-NP, LIFO-P, RM and EDF) were implemented in a single Python program, compare_lifo_rm_edf.py, listed in Appendix A. The simulator advances time in unit steps over one hyperperiod. At each step it (1) drops jobs whose deadline has passed and records them as misses, (2) releases new jobs, (3) asks the policy which ready job should run, and (4) executes that job for one time unit. Using one shared simulator guarantees that differences in the results come only from the scheduling rule.

TABLE 15  Simulation assumptions

| Assumption | Value used |
| --- | --- |
| Processor | One CPU |
| Time | Discrete, unit steps |
| Simulation length | One hyperperiod (LCM of periods) |
| First release | All tasks at time 0 (critical instant) |
| Deadlines | Implicit: Dᵢ = Tᵢ |
| Late jobs | Dropped at the deadline and counted as a miss |
| Context-switch cost | Zero |
| Ties in LIFO | Higher task index is treated as the later arrival |
| Ties in RM | Lower task index wins |
| Ties in EDF | Running job keeps the CPU, then lower task index |

## 7.2 Metrics

TABLE 16  Metrics reported for each run

| Metric | Definition | Better when |
| --- | --- | --- |
| Jobs | Jobs whose deadline lies inside the hyperperiod | - |
| Missed / Miss % | Jobs not finished by their deadline | Lower |
| Avg R / Max R | Average and maximum response time of completed jobs | Lower |
| Avg wait | Average time from release until first execution | Lower |
| Preemptions | Times an unfinished job lost the CPU | Lower |
| CPU % | Share of time the CPU did useful work | Equal to U when no job is dropped |

Response-time averages include only completed jobs. An algorithm that drops many jobs can therefore show a deceptively low average response time; the miss count must always be read first.

## 7.3 Scenarios

TABLE 17  The six task sets

| Scenario | Tasks (C, T) | n | U | H |
| --- | --- | --- | --- | --- |
| S1 Light load | T1(1,4) T2(1,6) T3(2,12) | 3 | 0.583 | 12 |
| S2 Above the RM bound | T1(1,4) T2(2,6) T3(3,12) | 3 | 0.833 | 12 |
| S3 High load | T1(2,5) T2(2,8) T3(3,10) | 3 | 0.950 | 40 |
| S4 Harmonic full load | T1(2,4) T2(2,8) T3(4,16) | 3 | 1.000 | 16 |
| S5 Five tasks | T1(1,5) T2(1,6) T3(2,10) T4(2,15) T5(3,30) | 5 | 0.800 | 30 |
| S6 Overload | T1(2,4) T2(3,6) T3(2,8) | 3 | 1.250 | 24 |

The scenarios were chosen to cover the whole range of load: below the Liu & Layland bound (S1), above it but still RM-schedulable (S2, S5), above RM’s exact limit but within EDF’s (S3), a harmonic set at full load (S4), and an overloaded system (S6).

# 8. Step-by-Step Worked Example

This section follows Scenario S2, tasks T1(1, 4), T2(2, 6) and T3(3, 12), one time unit at a time, so that every decision of every algorithm can be checked by hand. In the "Ready jobs" column, each job is followed by its absolute deadline in brackets.

## 8.1 LIFO-NP

TABLE 18  Decision trace of LIFO-NP for Scenario S2

| t | Ready jobs (deadline) | Runs | Reason for the decision |
| --- | --- | --- | --- |
| 0 | T1.1(4) T2.1(6) T3.1(12) | T3.1 | most recently released job (released at 0) |
| 1 | T1.1(4) T2.1(6) T3.1(12) | T3.1 | non-preemptive: current job keeps the CPU |
| 2 | T1.1(4) T2.1(6) T3.1(12) | T3.1 | non-preemptive: current job keeps the CPU |
| 3 | T1.1(4) T2.1(6) | T2.1 | most recently released job (released at 0) |
| 4 | T2.1(6) T1.2(8) | T2.1 | non-preemptive: current job keeps the CPU |
| 5 | T1.2(8) | T1.2 | most recently released job (released at 4) |
| 6 | T2.2(12) | T2.2 | most recently released job (released at 6) |
| 7 | T2.2(12) | T2.2 | non-preemptive: current job keeps the CPU |
| 8 | T1.3(12) | T1.3 | most recently released job (released at 8) |
| 9 | - | idle | no ready job: CPU idle |
| 10 | - | idle | no ready job: CPU idle |
| 11 | - | idle | no ready job: CPU idle |

## 8.2 LIFO-P

TABLE 19  Decision trace of LIFO-P for Scenario S2

| t | Ready jobs (deadline) | Runs | Reason for the decision |
| --- | --- | --- | --- |
| 0 | T1.1(4) T2.1(6) T3.1(12) | T3.1 | most recently released job (released at 0) |
| 1 | T1.1(4) T2.1(6) T3.1(12) | T3.1 | most recently released job (released at 0), continues |
| 2 | T1.1(4) T2.1(6) T3.1(12) | T3.1 | most recently released job (released at 0), continues |
| 3 | T1.1(4) T2.1(6) | T2.1 | most recently released job (released at 0) |
| 4 | T2.1(6) T1.2(8) | T1.2 | most recently released job (released at 4) |
| 5 | T2.1(6) | T2.1 | most recently released job (released at 0) |
| 6 | T2.2(12) | T2.2 | most recently released job (released at 6) |
| 7 | T2.2(12) | T2.2 | most recently released job (released at 6), continues |
| 8 | T1.3(12) | T1.3 | most recently released job (released at 8) |
| 9 | - | idle | no ready job: CPU idle |
| 10 | - | idle | no ready job: CPU idle |
| 11 | - | idle | no ready job: CPU idle |

## 8.3 RM

TABLE 20  Decision trace of RM for Scenario S2

| t | Ready jobs (deadline) | Runs | Reason for the decision |
| --- | --- | --- | --- |
| 0 | T1.1(4) T2.1(6) T3.1(12) | T1.1 | shortest period among ready jobs (T = 4) |
| 1 | T2.1(6) T3.1(12) | T2.1 | shortest period among ready jobs (T = 6) |
| 2 | T2.1(6) T3.1(12) | T2.1 | shortest period among ready jobs (T = 6), continues |
| 3 | T3.1(12) | T3.1 | shortest period among ready jobs (T = 12) |
| 4 | T3.1(12) T1.2(8) | T1.2 | shortest period among ready jobs (T = 4) |
| 5 | T3.1(12) | T3.1 | shortest period among ready jobs (T = 12) |
| 6 | T3.1(12) T2.2(12) | T2.2 | shortest period among ready jobs (T = 6) |
| 7 | T3.1(12) T2.2(12) | T2.2 | shortest period among ready jobs (T = 6), continues |
| 8 | T3.1(12) T1.3(12) | T1.3 | shortest period among ready jobs (T = 4) |
| 9 | T3.1(12) | T3.1 | shortest period among ready jobs (T = 12) |
| 10 | - | idle | no ready job: CPU idle |
| 11 | - | idle | no ready job: CPU idle |

## 8.4 EDF

TABLE 21  Decision trace of EDF for Scenario S2

| t | Ready jobs (deadline) | Runs | Reason for the decision |
| --- | --- | --- | --- |
| 0 | T1.1(4) T2.1(6) T3.1(12) | T1.1 | earliest absolute deadline (d = 4) |
| 1 | T2.1(6) T3.1(12) | T2.1 | earliest absolute deadline (d = 6) |
| 2 | T2.1(6) T3.1(12) | T2.1 | earliest absolute deadline (d = 6), continues |
| 3 | T3.1(12) | T3.1 | earliest absolute deadline (d = 12) |
| 4 | T3.1(12) T1.2(8) | T1.2 | earliest absolute deadline (d = 8) |
| 5 | T3.1(12) | T3.1 | earliest absolute deadline (d = 12) |
| 6 | T3.1(12) T2.2(12) | T3.1 | earliest absolute deadline (d = 12), continues |
| 7 | T2.2(12) | T2.2 | earliest absolute deadline (d = 12) |
| 8 | T2.2(12) T1.3(12) | T2.2 | earliest absolute deadline (d = 12), continues |
| 9 | T1.3(12) | T1.3 | earliest absolute deadline (d = 12) |
| 10 | - | idle | no ready job: CPU idle |
| 11 | - | idle | no ready job: CPU idle |

## 8.5 What the traces show

At time 0, all three tasks are released together. Both LIFO variants pick T3.1 because, under the tie-breaking rule, it is the last job pushed on the stack. T3 needs three units, and T2 is the next most recent job, so T1.1 does not get the CPU before its deadline at time 4 and is dropped. The miss is caused purely by arrival order: T1.1 is the most urgent job in the system, but LIFO has no way to know that.

RM starts with T1.1 because T1 has the shortest period, then serves T2 and finally T3. EDF makes the same choices at the start because the job with the shortest period also has the earliest deadline at time 0. The two algorithms diverge at time 6: RM gives T2.2 (period 6) priority over T3.1, while EDF sees that T3.1 (deadline 12) and T2.2 (deadline 12) are equally urgent and keeps running T3.1. Both meet every deadline.

# 9. Experimental Results

This section presents the results of the six scenarios. For each scenario it shows the task set and its analytical tests, the overall results, the jobs that missed their deadline, per-task statistics, and the full execution timeline. Each cell of a timeline shows the task running in that time unit.

## 9.1 Scenario S1: light load (U = 0.583)

S1 is well below the Liu & Layland bound for three tasks (0.780), so RM and EDF are guaranteed to meet every deadline. The question is whether LIFO also succeeds when the processor is mostly idle.

TABLE 22  Task set of S1 (hyperperiod = 12)

| Task | C (WCET) | T (period) | D (deadline) | U = C/T | RM priority | RTA response R | R ≤ D ? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 1 | 4 | 4 | 0.250 | 1 | 1 | Yes |
| T2 | 1 | 6 | 6 | 0.167 | 2 | 2 | Yes |
| T3 | 2 | 12 | 12 | 0.167 | 3 | 4 | Yes |
| Total |  |  |  | 0.583 |  |  |  |

TABLE 23  Schedulability tests for S1

| Test | Computed value | Limit | Result |
| --- | --- | --- | --- |
| Liu & Layland bound (RM) | U = 0.583 | 0.780 (n = 3) | Pass |
| Hyperbolic bound (RM) | ∏(Uᵢ + 1) = 1.701 | 2.000 | Pass |
| Response Time Analysis (RM) | all Rᵢ ≤ Dᵢ | exact | Pass |
| Utilization test (EDF) | U = 0.583 | 1.000 | Pass |
| Any test (LIFO) | no analytical test exists | - | Not available |

TABLE 24  Overall simulation results for S1

| Algorithm | Jobs | Done | Missed | Miss % | Avg R | Max R | Avg wait | Preempt. | CPU % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | 6 | 6 | 0 | 0.0 | 2.00 | 4 | 0.83 | 0 | 58.3 |
| LIFO-P | 6 | 6 | 0 | 0.0 | 2.00 | 4 | 0.83 | 0 | 58.3 |
| RM | 6 | 6 | 0 | 0.0 | 1.67 | 4 | 0.50 | 0 | 58.3 |
| EDF | 6 | 6 | 0 | 0.0 | 1.67 | 4 | 0.50 | 0 | 58.3 |

TABLE 25  Deadline misses in S1

| Algorithm | Jobs that missed their deadline (job @ absolute deadline) |
| --- | --- |
| LIFO-NP | none |
| LIFO-P | none |
| RM | none |
| EDF | none |

TABLE 26  Per-task results in S1 (misses | average response | maximum response)

| Task | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- |
| T1 | miss 0 ¦ avg 2.0 ¦ max 4 | miss 0 ¦ avg 2.0 ¦ max 4 | miss 0 ¦ avg 1.0 ¦ max 1 | miss 0 ¦ avg 1.0 ¦ max 1 |
| T2 | miss 0 ¦ avg 2.0 ¦ max 3 | miss 0 ¦ avg 2.0 ¦ max 3 | miss 0 ¦ avg 1.5 ¦ max 2 | miss 0 ¦ avg 1.5 ¦ max 2 |
| T3 | miss 0 ¦ avg 2.0 ¦ max 2 | miss 0 ¦ avg 2.0 ¦ max 2 | miss 0 ¦ avg 4.0 ¦ max 4 | miss 0 ¦ avg 4.0 ¦ max 4 |

TABLE 27  Execution timeline of S1 (· = idle)

| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T2 | T1 | T1 | · | T2 | · | T1 | · | · | · |
| LIFO-P | T3 | T3 | T2 | T1 | T1 | · | T2 | · | T1 | · | · | · |
| RM | T1 | T2 | T3 | T3 | T1 | · | T2 | · | T1 | · | · | · |
| EDF | T1 | T2 | T3 | T3 | T1 | · | T2 | · | T1 | · | · | · |

### Discussion

All four algorithms meet all deadlines, which supports hypothesis H1. The difference lies in how close they come to failure. Both LIFO variants start with T3 (two units) and then T2, so T1.1 runs in the last possible slot and finishes at exactly time 4, its deadline. Its response time equals its period.

RM and EDF run T1 first and produce identical schedules with an average response time of 1.67 instead of 2.00. They also give the most urgent task a response time of 1, leaving a safety margin of three units. LIFO met its deadlines here by luck, not by design: any extra unit of execution in T2 or T3 would have caused a miss.

KEY OBSERVATION Even at 58 % load, LIFO left zero slack for the most urgent task. RM and EDF met the same deadlines with a large safety margin.

## 9.2 Scenario S2: above the RM bound (U = 0.833)

With U = 0.833, S2 is above the Liu & Layland bound, so that test cannot guarantee RM. The hyperbolic bound (2.083 > 2) is also inconclusive. Only the exact RTA shows that RM is safe, with a worst-case response of 10 for T3 against a deadline of 12.

TABLE 28  Task set of S2 (hyperperiod = 12)

| Task | C (WCET) | T (period) | D (deadline) | U = C/T | RM priority | RTA response R | R ≤ D ? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 1 | 4 | 4 | 0.250 | 1 | 1 | Yes |
| T2 | 2 | 6 | 6 | 0.333 | 2 | 3 | Yes |
| T3 | 3 | 12 | 12 | 0.250 | 3 | 10 | Yes |
| Total |  |  |  | 0.833 |  |  |  |

TABLE 29  Schedulability tests for S2

| Test | Computed value | Limit | Result |
| --- | --- | --- | --- |
| Liu & Layland bound (RM) | U = 0.833 | 0.780 (n = 3) | Inconclusive |
| Hyperbolic bound (RM) | ∏(Uᵢ + 1) = 2.083 | 2.000 | Inconclusive |
| Response Time Analysis (RM) | all Rᵢ ≤ Dᵢ | exact | Pass |
| Utilization test (EDF) | U = 0.833 | 1.000 | Pass |
| Any test (LIFO) | no analytical test exists | - | Not available |

TABLE 30  Overall simulation results for S2

| Algorithm | Jobs | Done | Missed | Miss % | Avg R | Max R | Avg wait | Preempt. | CPU % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | 6 | 5 | 1 | 16.7 | 2.60 | 5 | 0.80 | 0 | 75.0 |
| LIFO-P | 6 | 5 | 1 | 16.7 | 2.60 | 6 | 0.60 | 1 | 75.0 |
| RM | 6 | 6 | 0 | 0.0 | 3.00 | 10 | 0.67 | 2 | 83.3 |
| EDF | 6 | 6 | 0 | 0.0 | 2.83 | 7 | 1.00 | 1 | 83.3 |

TABLE 31  Deadline misses in S2

| Algorithm | Jobs that missed their deadline (job @ absolute deadline) |
| --- | --- |
| LIFO-NP | T1.1 @ 4 |
| LIFO-P | T1.1 @ 4 |
| RM | none |
| EDF | none |

TABLE 32  Per-task results in S2 (misses | average response | maximum response)

| Task | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- |
| T1 | miss 1 ¦ avg 1.5 ¦ max 2 | miss 1 ¦ avg 1.0 ¦ max 1 | miss 0 ¦ avg 1.0 ¦ max 1 | miss 0 ¦ avg 1.3 ¦ max 2 |
| T2 | miss 0 ¦ avg 3.5 ¦ max 5 | miss 0 ¦ avg 4.0 ¦ max 6 | miss 0 ¦ avg 2.5 ¦ max 3 | miss 0 ¦ avg 3.0 ¦ max 3 |
| T3 | miss 0 ¦ avg 3.0 ¦ max 3 | miss 0 ¦ avg 3.0 ¦ max 3 | miss 0 ¦ avg 10.0 ¦ max 10 | miss 0 ¦ avg 7.0 ¦ max 7 |

TABLE 33  Execution timeline of S2 (· = idle)

| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T3 | T2 | T2 | T1 | T2 | T2 | T1 | · | · | · |
| LIFO-P | T3 | T3 | T3 | T2 | T1 | T2 | T2 | T2 | T1 | · | · | · |
| RM | T1 | T2 | T2 | T3 | T1 | T3 | T2 | T2 | T1 | T3 | · | · |
| EDF | T1 | T2 | T2 | T3 | T1 | T3 | T3 | T2 | T2 | T1 | · | · |

### Discussion

RM and EDF meet every deadline, confirming that the Liu & Layland test is only sufficient. Both LIFO variants miss T1.1, as explained step by step in Section 8: three units of T3 and two units of T2 run before T1 is ever considered.

LIFO shows a lower average response time (2.60) than RM (3.00) and EDF (2.83), but this number is misleading because the dropped job is excluded. RM has the largest maximum response time (10, for T3) because T3 is its lowest-priority task; EDF reduces this to 7 by letting T3 run when its deadline becomes the earliest.

KEY OBSERVATION A single wrongly ordered decision at time 0 is enough for LIFO to miss a deadline, at a load where both real-time algorithms succeed.

## 9.3 Scenario S3: high load (U = 0.950)

S3 is the scenario that separates RM from EDF. Its utilization is below 1, so EDF is guaranteed to succeed, but RTA shows that the worst-case response of T3 under RM is 11, larger than its deadline of 10.

TABLE 34  Task set of S3 (hyperperiod = 40)

| Task | C (WCET) | T (period) | D (deadline) | U = C/T | RM priority | RTA response R | R ≤ D ? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 2 | 5 | 5 | 0.400 | 1 | 2 | Yes |
| T2 | 2 | 8 | 8 | 0.250 | 2 | 4 | Yes |
| T3 | 3 | 10 | 10 | 0.300 | 3 | 11 | No |
| Total |  |  |  | 0.950 |  |  |  |

TABLE 35  Schedulability tests for S3

| Test | Computed value | Limit | Result |
| --- | --- | --- | --- |
| Liu & Layland bound (RM) | U = 0.950 | 0.780 (n = 3) | Inconclusive |
| Hyperbolic bound (RM) | ∏(Uᵢ + 1) = 2.275 | 2.000 | Inconclusive |
| Response Time Analysis (RM) | some Rᵢ > Dᵢ | exact | Fail |
| Utilization test (EDF) | U = 0.950 | 1.000 | Pass |
| Any test (LIFO) | no analytical test exists | - | Not available |

TABLE 36  Overall simulation results for S3

| Algorithm | Jobs | Done | Missed | Miss % | Avg R | Max R | Avg wait | Preempt. | CPU % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | 17 | 15 | 2 | 11.8 | 3.20 | 5 | 0.93 | 0 | 85.0 |
| LIFO-P | 17 | 14 | 3 | 17.6 | 3.14 | 5 | 0.43 | 4 | 82.5 |
| RM | 17 | 16 | 1 | 5.9 | 3.38 | 9 | 0.69 | 5 | 92.5 |
| EDF | 17 | 17 | 0 | 0.0 | 3.88 | 7 | 1.53 | 1 | 95.0 |

TABLE 37  Deadline misses in S3

| Algorithm | Jobs that missed their deadline (job @ absolute deadline) |
| --- | --- |
| LIFO-NP | T1.1 @ 5, T1.7 @ 35 |
| LIFO-P | T1.1 @ 5, T1.5 @ 25, T1.7 @ 35 |
| RM | T3.1 @ 10 |
| EDF | none |

TABLE 38  Per-task results in S3 (misses | average response | maximum response)

| Task | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- |
| T1 | miss 2 ¦ avg 3.0 ¦ max 5 | miss 3 ¦ avg 3.0 ¦ max 5 | miss 0 ¦ avg 2.0 ¦ max 2 | miss 0 ¦ avg 2.6 ¦ max 4 |
| T2 | miss 0 ¦ avg 3.6 ¦ max 5 | miss 0 ¦ avg 3.0 ¦ max 5 | miss 0 ¦ avg 3.0 ¦ max 4 | miss 0 ¦ avg 4.0 ¦ max 5 |
| T3 | miss 0 ¦ avg 3.0 ¦ max 3 | miss 0 ¦ avg 3.5 ¦ max 5 | miss 1 ¦ avg 7.7 ¦ max 9 | miss 0 ¦ avg 6.3 ¦ max 7 |

TABLE 39  Execution timeline of S3 (· = idle)

| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T3 | T2 | T2 | T1 | T1 | · | T2 | T2 | T3 | T3 | T3 | T1 | T1 | T1 | T1 | T2 | T2 | · |
| LIFO-P | T3 | T3 | T3 | T2 | T2 | T1 | T1 | · | T2 | T2 | T3 | T3 | T3 | T1 | T1 | T1 | T2 | T2 | T1 | · |
| RM | T1 | T1 | T2 | T2 | T3 | T1 | T1 | T3 | T2 | T2 | T1 | T1 | T3 | T3 | T3 | T1 | T1 | T2 | T2 | · |
| EDF | T1 | T1 | T2 | T2 | T3 | T3 | T3 | T1 | T1 | T2 | T1 | T1 | T2 | T3 | T3 | T3 | T1 | T1 | T2 | T2 |

| t | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T3 | T1 | T1 | T1 | T1 | T2 | T2 | · | T3 | T3 | T3 | T2 | T2 | T1 | T1 | · | · | · |
| LIFO-P | T3 | T3 | T3 | T1 | T2 | T1 | T1 | T2 | · | · | T3 | T3 | T2 | T2 | T3 | T1 | T1 | · | · | · |
| RM | T1 | T1 | T3 | T3 | T2 | T1 | T1 | T2 | T3 | · | T1 | T1 | T2 | T2 | T3 | T1 | T1 | T3 | T3 | · |
| EDF | T1 | T1 | T3 | T3 | T3 | T1 | T1 | T2 | T2 | · | T1 | T1 | T2 | T2 | T3 | T3 | T3 | T1 | T1 | · |

### Discussion

EDF meets all 17 deadlines with only one preemption and keeps the CPU 95 % busy, exactly equal to U. RM misses exactly the job predicted by RTA, T3.1 at time 10, and recovers afterwards. This confirms hypothesis H3 and shows that RTA predictions match simulation precisely.

LIFO-NP misses two jobs and LIFO-P three, and every one of them belongs to T1, the most frequent task. Preemptive LIFO is worse than non-preemptive LIFO here, because each new release of T2 or T3 interrupts T1, pushing T1 deeper into the stack. LIFO-P also has the lowest average waiting time (0.43), which shows again that response-oriented metrics can hide deadline failures.

KEY OBSERVATION At 95 % load only EDF meets every deadline. RM fails exactly where its exact test predicts. LIFO fails repeatedly, always on the most urgent task.

## 9.4 Scenario S4: harmonic set at full load (U = 1.000)

S4 uses harmonic periods (4, 8 and 16) and fills the processor completely. The Liu & Layland bound is far exceeded, yet RTA shows that RM is schedulable, with T3 finishing exactly at its deadline.

TABLE 40  Task set of S4 (hyperperiod = 16)

| Task | C (WCET) | T (period) | D (deadline) | U = C/T | RM priority | RTA response R | R ≤ D ? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 2 | 4 | 4 | 0.500 | 1 | 2 | Yes |
| T2 | 2 | 8 | 8 | 0.250 | 2 | 4 | Yes |
| T3 | 4 | 16 | 16 | 0.250 | 3 | 16 | Yes |
| Total |  |  |  | 1.000 |  |  |  |

TABLE 41  Schedulability tests for S4

| Test | Computed value | Limit | Result |
| --- | --- | --- | --- |
| Liu & Layland bound (RM) | U = 1.000 | 0.780 (n = 3) | Inconclusive |
| Hyperbolic bound (RM) | ∏(Uᵢ + 1) = 2.344 | 2.000 | Inconclusive |
| Response Time Analysis (RM) | all Rᵢ ≤ Dᵢ | exact | Pass |
| Utilization test (EDF) | U = 1.000 | 1.000 | Pass |
| Any test (LIFO) | no analytical test exists | - | Not available |

TABLE 42  Overall simulation results for S4

| Algorithm | Jobs | Done | Missed | Miss % | Avg R | Max R | Avg wait | Preempt. | CPU % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | 7 | 6 | 1 | 14.3 | 3.67 | 8 | 1.33 | 0 | 87.5 |
| LIFO-P | 7 | 6 | 1 | 14.3 | 3.67 | 8 | 1.33 | 0 | 87.5 |
| RM | 7 | 7 | 0 | 0.0 | 4.57 | 16 | 1.43 | 1 | 100.0 |
| EDF | 7 | 7 | 0 | 0.0 | 4.57 | 16 | 1.43 | 1 | 100.0 |

TABLE 43  Deadline misses in S4

| Algorithm | Jobs that missed their deadline (job @ absolute deadline) |
| --- | --- |
| LIFO-NP | T1.1 @ 4 |
| LIFO-P | T1.1 @ 4 |
| RM | none |
| EDF | none |

TABLE 44  Per-task results in S4 (misses | average response | maximum response)

| Task | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- |
| T1 | miss 1 ¦ avg 2.7 ¦ max 4 | miss 1 ¦ avg 2.7 ¦ max 4 | miss 0 ¦ avg 2.0 ¦ max 2 | miss 0 ¦ avg 2.0 ¦ max 2 |
| T2 | miss 0 ¦ avg 5.0 ¦ max 8 | miss 0 ¦ avg 5.0 ¦ max 8 | miss 0 ¦ avg 4.0 ¦ max 4 | miss 0 ¦ avg 4.0 ¦ max 4 |
| T3 | miss 0 ¦ avg 4.0 ¦ max 4 | miss 0 ¦ avg 4.0 ¦ max 4 | miss 0 ¦ avg 16.0 ¦ max 16 | miss 0 ¦ avg 16.0 ¦ max 16 |

TABLE 45  Execution timeline of S4 (· = idle)

| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T3 | T3 | T1 | T1 | T2 | T2 | T2 | T2 | T1 | T1 | T1 | T1 | · | · |
| LIFO-P | T3 | T3 | T3 | T3 | T1 | T1 | T2 | T2 | T2 | T2 | T1 | T1 | T1 | T1 | · | · |
| RM | T1 | T1 | T2 | T2 | T1 | T1 | T3 | T3 | T1 | T1 | T2 | T2 | T1 | T1 | T3 | T3 |
| EDF | T1 | T1 | T2 | T2 | T1 | T1 | T3 | T3 | T1 | T1 | T2 | T2 | T1 | T1 | T3 | T3 |

### Discussion

RM and EDF produce the same schedule, meet all seven deadlines and keep the CPU busy 100 % of the time, confirming hypothesis H4. Under harmonic periods, the shortest-period job always has the earliest deadline, so the two rules choose the same job at every step.

Both LIFO variants miss T1.1 because T3 occupies the first four units. Since the dropped job’s work is never done, the processor sits idle for the last two units, and CPU utilization drops to 87.5 %. On a fully loaded system, every wasted slot means lost work that cannot be recovered.

KEY OBSERVATION With harmonic periods RM is as good as EDF and uses 100 % of the CPU. LIFO wastes capacity even when the task set is perfectly schedulable.

## 9.5 Scenario S5: five tasks (U = 0.800)

S5 increases the number of tasks to five. The Liu & Layland bound drops to 0.743 and is exceeded, but RTA proves that RM is schedulable. The longest task, T5 (C = 3, T = 30), is released together with all others at time 0.

TABLE 46  Task set of S5 (hyperperiod = 30)

| Task | C (WCET) | T (period) | D (deadline) | U = C/T | RM priority | RTA response R | R ≤ D ? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 1 | 5 | 5 | 0.200 | 1 | 1 | Yes |
| T2 | 1 | 6 | 6 | 0.167 | 2 | 2 | Yes |
| T3 | 2 | 10 | 10 | 0.200 | 3 | 4 | Yes |
| T4 | 2 | 15 | 15 | 0.133 | 4 | 8 | Yes |
| T5 | 3 | 30 | 30 | 0.100 | 5 | 15 | Yes |
| Total |  |  |  | 0.800 |  |  |  |

TABLE 47  Schedulability tests for S5

| Test | Computed value | Limit | Result |
| --- | --- | --- | --- |
| Liu & Layland bound (RM) | U = 0.800 | 0.743 (n = 5) | Inconclusive |
| Hyperbolic bound (RM) | ∏(Uᵢ + 1) = 2.094 | 2.000 | Inconclusive |
| Response Time Analysis (RM) | all Rᵢ ≤ Dᵢ | exact | Pass |
| Utilization test (EDF) | U = 0.800 | 1.000 | Pass |
| Any test (LIFO) | no analytical test exists | - | Not available |

TABLE 48  Overall simulation results for S5

| Algorithm | Jobs | Done | Missed | Miss % | Avg R | Max R | Avg wait | Preempt. | CPU % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | 17 | 15 | 2 | 11.8 | 2.60 | 9 | 1.13 | 0 | 73.3 |
| LIFO-P | 17 | 15 | 2 | 11.8 | 2.60 | 9 | 1.13 | 0 | 73.3 |
| RM | 17 | 17 | 0 | 0.0 | 2.88 | 15 | 1.06 | 3 | 80.0 |
| EDF | 17 | 17 | 0 | 0.0 | 2.88 | 15 | 1.06 | 3 | 80.0 |

TABLE 49  Deadline misses in S5

| Algorithm | Jobs that missed their deadline (job @ absolute deadline) |
| --- | --- |
| LIFO-NP | T1.1 @ 5, T2.1 @ 6 |
| LIFO-P | T1.1 @ 5, T2.1 @ 6 |
| RM | none |
| EDF | none |

TABLE 50  Per-task results in S5 (misses | average response | maximum response)

| Task | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- |
| T1 | miss 1 ¦ avg 2.4 ¦ max 4 | miss 1 ¦ avg 2.4 ¦ max 4 | miss 0 ¦ avg 1.0 ¦ max 1 | miss 0 ¦ avg 1.0 ¦ max 1 |
| T2 | miss 1 ¦ avg 1.0 ¦ max 1 | miss 1 ¦ avg 1.0 ¦ max 1 | miss 0 ¦ avg 1.2 ¦ max 2 | miss 0 ¦ avg 1.2 ¦ max 2 |
| T3 | miss 0 ¦ avg 4.3 ¦ max 9 | miss 0 ¦ avg 4.3 ¦ max 9 | miss 0 ¦ avg 3.7 ¦ max 4 | miss 0 ¦ avg 3.7 ¦ max 4 |
| T4 | miss 0 ¦ avg 3.5 ¦ max 5 | miss 0 ¦ avg 3.5 ¦ max 5 | miss 0 ¦ avg 5.5 ¦ max 8 | miss 0 ¦ avg 5.5 ¦ max 8 |
| T5 | miss 0 ¦ avg 3.0 ¦ max 3 | miss 0 ¦ avg 3.0 ¦ max 3 | miss 0 ¦ avg 15.0 ¦ max 15 | miss 0 ¦ avg 15.0 ¦ max 15 |

TABLE 51  Execution timeline of S5 (· = idle)

| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T5 | T5 | T5 | T4 | T4 | T1 | T2 | T3 | T3 | · | T3 | T3 | T2 | T1 | · | T4 | T4 | T1 | T2 | · |
| LIFO-P | T5 | T5 | T5 | T4 | T4 | T1 | T2 | T3 | T3 | · | T3 | T3 | T2 | T1 | · | T4 | T4 | T1 | T2 | · |
| RM | T1 | T2 | T3 | T3 | T4 | T1 | T2 | T4 | T5 | T5 | T1 | T3 | T2 | T3 | T5 | T1 | T4 | T4 | T2 | · |
| EDF | T1 | T2 | T3 | T3 | T4 | T1 | T2 | T4 | T5 | T5 | T1 | T3 | T2 | T3 | T5 | T1 | T4 | T4 | T2 | · |

| t | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T1 | · | T2 | T1 | · | · | · | · |
| LIFO-P | T3 | T3 | T1 | · | T2 | T1 | · | · | · | · |
| RM | T1 | T3 | T3 | · | T2 | T1 | · | · | · | · |
| EDF | T1 | T3 | T3 | · | T2 | T1 | · | · | · | · |

### Discussion

RM and EDF again produce identical schedules with no misses. Both LIFO variants start with T5, the task with the longest period and therefore the least urgent one, and then T4. As a result the two most urgent tasks, T1 and T2, both miss their first deadline.

This scenario shows how LIFO inverts urgency when many tasks are released together: the task listed last, here the slowest one, is treated as the newest arrival. With more tasks, the stack becomes deeper and the jobs at the bottom wait longer.

KEY OBSERVATION The more tasks there are, the further LIFO’s order can drift from the urgency order. Five tasks at 80 % load are enough to make LIFO miss two deadlines.

## 9.6 Scenario S6: overload (U = 1.250)

S6 demands 25 % more processing than the CPU can provide, so no algorithm can meet all deadlines. The interesting question is which jobs each algorithm sacrifices.

TABLE 52  Task set of S6 (hyperperiod = 24)

| Task | C (WCET) | T (period) | D (deadline) | U = C/T | RM priority | RTA response R | R ≤ D ? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 2 | 4 | 4 | 0.500 | 1 | 2 | Yes |
| T2 | 3 | 6 | 6 | 0.500 | 2 | 7 | No |
| T3 | 2 | 8 | 8 | 0.250 | 3 | 12 | No |
| Total |  |  |  | 1.250 |  |  |  |

TABLE 53  Schedulability tests for S6

| Test | Computed value | Limit | Result |
| --- | --- | --- | --- |
| Liu & Layland bound (RM) | U = 1.250 | 0.780 (n = 3) | Inconclusive |
| Hyperbolic bound (RM) | ∏(Uᵢ + 1) = 2.813 | 2.000 | Inconclusive |
| Response Time Analysis (RM) | some Rᵢ > Dᵢ | exact | Fail |
| Utilization test (EDF) | U = 1.250 | 1.000 | Fail |
| Any test (LIFO) | no analytical test exists | - | Not available |

TABLE 54  Overall simulation results for S6

| Algorithm | Jobs | Done | Missed | Miss % | Avg R | Max R | Avg wait | Preempt. | CPU % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | 13 | 9 | 4 | 30.8 | 3.22 | 5 | 0.78 | 0 | 95.8 |
| LIFO-P | 13 | 8 | 5 | 38.5 | 2.75 | 5 | 0.25 | 3 | 95.8 |
| RM | 13 | 8 | 5 | 38.5 | 2.75 | 5 | 0.00 | 5 | 100.0 |
| EDF | 13 | 8 | 5 | 38.5 | 3.50 | 8 | 1.38 | 0 | 100.0 |

TABLE 55  Deadline misses in S6

| Algorithm | Jobs that missed their deadline (job @ absolute deadline) |
| --- | --- |
| LIFO-NP | T1.1 @ 4, T1.3 @ 12, T1.4 @ 16, T1.5 @ 20 |
| LIFO-P | T1.1 @ 4, T2.1 @ 6, T2.2 @ 12, T1.4 @ 16, T1.5 @ 20 |
| RM | T2.1 @ 6, T3.1 @ 8, T3.2 @ 16, T2.3 @ 18, T3.3 @ 24 |
| EDF | T3.1 @ 8, T2.2 @ 12, T2.3 @ 18, T3.3 @ 24, T2.4 @ 24 |

TABLE 56  Per-task results in S6 (misses | average response | maximum response)

| Task | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- |
| T1 | miss 4 ¦ avg 3.0 ¦ max 3 | miss 3 ¦ avg 2.7 ¦ max 4 | miss 0 ¦ avg 2.0 ¦ max 2 | miss 0 ¦ avg 2.5 ¦ max 4 |
| T2 | miss 0 ¦ avg 3.8 ¦ max 5 | miss 2 ¦ avg 4.0 ¦ max 5 | miss 2 ¦ avg 5.0 ¦ max 5 | miss 3 ¦ avg 5.0 ¦ max 5 |
| T3 | miss 0 ¦ avg 2.7 ¦ max 4 | miss 0 ¦ avg 2.0 ¦ max 2 | miss 3 ¦ avg - ¦ max - | miss 2 ¦ avg 8.0 ¦ max 8 |

TABLE 57  Execution timeline of S6 (· = idle)

| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LIFO-NP | T3 | T3 | T2 | T2 | T2 | T1 | T1 | T2 | T2 | T2 | T3 | T3 | T2 | T2 | T2 | T1 | T3 | T3 | T2 | T2 |
| LIFO-P | T3 | T3 | T2 | T2 | T1 | T1 | T2 | T2 | T3 | T3 | T1 | T1 | T2 | T2 | T2 | T1 | T3 | T3 | T2 | T2 |
| RM | T1 | T1 | T2 | T2 | T1 | T1 | T2 | T2 | T1 | T1 | T2 | T3 | T1 | T1 | T2 | T2 | T1 | T1 | T2 | T2 |
| EDF | T1 | T1 | T2 | T2 | T2 | T1 | T1 | T3 | T1 | T1 | T2 | T2 | T1 | T1 | T3 | T3 | T2 | T2 | T1 | T1 |

| t | 20 | 21 | 22 | 23 |
| --- | --- | --- | --- | --- |
| LIFO-NP | T2 | T1 | T1 | · |
| LIFO-P | T1 | T1 | T2 | · |
| RM | T1 | T1 | T2 | T3 |
| EDF | T1 | T1 | T2 | T2 |

### Discussion

All algorithms miss deadlines, as expected (hypothesis H5). In raw numbers LIFO-NP misses the fewest jobs (4), but all four are jobs of T1, the task with the highest rate. In most real systems the highest-rate task is the control loop that matters most, so this is the worst possible choice of victim.

RM never misses a T1 job. It sacrifices T3 completely and T2 partly, which is the predictable and designed behaviour of fixed priorities: the system degrades from the bottom. EDF spreads the misses over T2 and T3 and, through the domino effect, misses two jobs at time 24 alone. RM and EDF keep the CPU 100 % busy, while LIFO wastes 4.2 %.

KEY OBSERVATION Under overload, the number of misses is not the full story. RM protects its most urgent task completely, EDF spreads the damage, and LIFO sacrifices the most urgent task first.

# 10. Summary Across Scenarios

The tables in this section collect the main metrics of all scenarios to show general trends.

TABLE 58  Deadline misses per scenario

| Scenario | U | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- | --- |
| S1 | 0.583 | 0 | 0 | 0 | 0 |
| S2 | 0.833 | 1 | 1 | 0 | 0 |
| S3 | 0.950 | 2 | 3 | 1 | 0 |
| S4 | 1.000 | 1 | 1 | 0 | 0 |
| S5 | 0.800 | 2 | 2 | 0 | 0 |
| S6 | 1.250 | 4 | 5 | 5 | 5 |

TABLE 59  Miss rate (%) per scenario

| Scenario | U | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- | --- |
| S1 | 0.583 | 0.0 | 0.0 | 0.0 | 0.0 |
| S2 | 0.833 | 16.7 | 16.7 | 0.0 | 0.0 |
| S3 | 0.950 | 11.8 | 17.6 | 5.9 | 0.0 |
| S4 | 1.000 | 14.3 | 14.3 | 0.0 | 0.0 |
| S5 | 0.800 | 11.8 | 11.8 | 0.0 | 0.0 |
| S6 | 1.250 | 30.8 | 38.5 | 38.5 | 38.5 |

TABLE 60  Average response time per scenario

| Scenario | U | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- | --- |
| S1 | 0.583 | 2.00 | 2.00 | 1.67 | 1.67 |
| S2 | 0.833 | 2.60 | 2.60 | 3.00 | 2.83 |
| S3 | 0.950 | 3.20 | 3.14 | 3.38 | 3.88 |
| S4 | 1.000 | 3.67 | 3.67 | 4.57 | 4.57 |
| S5 | 0.800 | 2.60 | 2.60 | 2.88 | 2.88 |
| S6 | 1.250 | 3.22 | 2.75 | 2.75 | 3.50 |

TABLE 61  Maximum response time per scenario

| Scenario | U | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- | --- |
| S1 | 0.583 | 4 | 4 | 4 | 4 |
| S2 | 0.833 | 5 | 6 | 10 | 7 |
| S3 | 0.950 | 5 | 5 | 9 | 7 |
| S4 | 1.000 | 8 | 8 | 16 | 16 |
| S5 | 0.800 | 9 | 9 | 15 | 15 |
| S6 | 1.250 | 5 | 5 | 5 | 8 |

TABLE 62  Preemptions per scenario

| Scenario | U | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- | --- |
| S1 | 0.583 | 0 | 0 | 0 | 0 |
| S2 | 0.833 | 0 | 1 | 2 | 1 |
| S3 | 0.950 | 0 | 4 | 5 | 1 |
| S4 | 1.000 | 0 | 0 | 1 | 1 |
| S5 | 0.800 | 0 | 0 | 3 | 3 |
| S6 | 1.250 | 0 | 3 | 5 | 0 |

TABLE 63  CPU utilization (%) per scenario

| Scenario | U | LIFO-NP | LIFO-P | RM | EDF |
| --- | --- | --- | --- | --- | --- |
| S1 | 0.583 | 58.3 | 58.3 | 58.3 | 58.3 |
| S2 | 0.833 | 75.0 | 75.0 | 83.3 | 83.3 |
| S3 | 0.950 | 85.0 | 82.5 | 92.5 | 95.0 |
| S4 | 1.000 | 87.5 | 87.5 | 100.0 | 100.0 |
| S5 | 0.800 | 73.3 | 73.3 | 80.0 | 80.0 |
| S6 | 1.250 | 95.8 | 95.8 | 100.0 | 100.0 |

TABLE 64  Number of scenarios with zero deadline misses

| Algorithm | Scenarios without misses | Out of feasible ones (S1 to S5) | Total misses (all scenarios) |
| --- | --- | --- | --- |
| LIFO-NP | 1 of 6 | 1 of 5 | 10 |
| LIFO-P | 1 of 6 | 1 of 5 | 12 |
| RM | 4 of 6 | 4 of 5 | 6 |
| EDF | 5 of 6 | 5 of 5 | 5 |

TABLE 65  Verification of the hypotheses

| # | Hypothesis | Result |
| --- | --- | --- |
| H1 | All algorithms succeed at light load. | Confirmed (S1), but LIFO had zero slack |
| H2 | Above the L&L bound RM can still succeed; LIFO fails. | Confirmed (S2, S5) |
| H3 | Between RM’s limit and U = 1 only EDF succeeds. | Confirmed (S3) |
| H4 | Harmonic sets let RM reach U = 1. | Confirmed (S4) |
| H5 | Under overload, victims differ by algorithm. | Confirmed (S6) |

TABLE 66  Overall ranking

| Rank | Algorithm | Justification |
| --- | --- | --- |
| 1 | EDF | Met every deadline in all five feasible scenarios; fewest preemptions; uses 100 % of the CPU. |
| 2 | RM | Met every deadline in four of five feasible scenarios; failed only where RTA predicted; best overload behaviour. |
| 3 | LIFO-NP | Missed deadlines in four of five feasible scenarios; no preemptions but no guarantees. |
| 4 | LIFO-P | Same failures as LIFO-NP and more misses in S3 and S6, because every release pushes urgent work deeper. |

# 11. Discussion

## 11.1 Why LIFO fails as a real-time scheduler

The experiments show a consistent pattern: LIFO misses deadlines in every scenario with a load of 80 % or more (S2 to S5). The root cause is that its priority comes from an attribute, arrival order, that has no relation to urgency. When several periodic tasks are released together, as happens at every multiple of their common periods, the order in which they are pushed on the stack decides everything. A slow, unimportant task can be served before a fast, critical one.

A second cause is that LIFO actively works against older jobs. In periodic systems, the job that has waited longest is usually the one closest to its deadline. LIFO serves it last. EDF does exactly the opposite and is optimal for that reason.

## 11.2 Preemptive versus non-preemptive LIFO

One might expect preemption to help, since it lets a newly released urgent job start immediately. In practice, preemptive LIFO lets every new release interrupt the running job, including releases of unimportant tasks. In S3 and S6, LIFO-P missed more deadlines than LIFO-NP. Preemption is only useful when it is guided by a meaningful priority, as in RM and EDF.

## 11.3 Misleading metrics

In several scenarios LIFO reported a lower average response or waiting time than RM or EDF. This is a statistical artefact: jobs that miss their deadline are dropped and do not count in the average, and LIFO tends to drop exactly the jobs that would have waited longest. For real-time systems the miss count must always be the primary metric, and response time is only meaningful between algorithms that miss nothing.

## 11.4 RM versus EDF

RM and EDF produced identical schedules in S1, S4 and S5, where the job with the shortest period also had the earliest deadline at every decision point. They differed in S2 (different order, both successful), in S3 (only EDF successful) and in S6 (different victims). EDF used fewer preemptions in S3 (1 against 5). RM’s advantage lies in predictability under overload: its victims are always the lowest-priority tasks, while EDF’s victims depend on the dynamics of the schedule.

## 11.5 Threats to validity

- The task sets are small and were constructed by hand; random task sets with many tasks would give more general statistics.

- Context-switch overhead is ignored. Including it would penalise LIFO-P and RM slightly more than EDF and LIFO-NP.

- Late jobs are dropped. If late jobs were allowed to continue, misses could propagate further, especially for EDF under overload.

- LIFO results depend on the tie-breaking rule. A different rule (for example pushing T3 first) would change which job misses, but not the fact that LIFO offers no guarantees.

# 12. Practical Recommendations

TABLE 67  Which algorithm to use

| Situation | Choice | Reason |
| --- | --- | --- |
| Safety-critical system that must be certified | RM | Fixed priorities are simple to verify; exact RTA available |
| Embedded controller with high CPU load | EDF | Schedulable up to U = 1; no wasted capacity |
| Task periods are harmonic | RM | Reaches U = 1 with fixed-priority simplicity |
| Overload is possible and critical tasks must survive | RM | Only the lowest-priority tasks are affected |
| Soft real-time multimedia or networking | EDF | Good average behaviour; misses tolerable |
| Parallel runtime with fork/join tasks | LIFO | Cache locality; no deadlines involved |
| Handling of nested interrupts | LIFO | Matches the natural nesting of handlers |
| Any task with a hard deadline | Not LIFO | No guarantee of any kind |

RULE OF THUMB Use LIFO only when tasks have no deadlines. For hard real-time work, choose RM when predictability and certification matter most, and EDF when the processor must be used to its full capacity.

# 13. Conclusion

This report compared LIFO, a simple stack-based policy, with the two classic real-time scheduling algorithms, Rate Monotonic and Earliest Deadline First, both analytically and through simulation of six periodic task sets.

The theory predicts a clear ranking, and the experiments confirmed it. EDF met every deadline in all five feasible scenarios, including S3 at 95 % load where RM failed, and it needed the fewest preemptions. RM succeeded in four of the five feasible scenarios; its only failure was exactly the one predicted by Response Time Analysis, and under overload it protected its most urgent task completely. LIFO, in both variants, missed deadlines in four of the five feasible scenarios, including one at only 83 % load, and in the overload scenario it sacrificed the most urgent task first.

LIFO is not a bad algorithm in general. Its O(1) decisions and its cache-friendly recency make it valuable in work-stealing runtimes and interrupt handling. But it has no concept of time, offers no schedulability test, and gives results that depend on accidental arrival order. For any system with deadlines, RM or EDF should be used, and the choice between them depends on whether predictability (RM) or full processor utilization (EDF) is more important.

# References

- C. L. Liu and J. W. Layland, "Scheduling Algorithms for Multiprogramming in a Hard-Real-Time Environment," Journal of the ACM, vol. 20, no. 1, pp. 46–61, 1973.

- M. L. Dertouzos, "Control Robotics: The Procedural Control of Physical Processes," Proceedings of the IFIP Congress, 1974.

- M. Joseph and P. Pandya, "Finding Response Times in a Real-Time System," The Computer Journal, vol. 29, no. 5, 1986.

- E. Bini, G. C. Buttazzo and G. M. Buttazzo, "Rate Monotonic Analysis: The Hyperbolic Bound," IEEE Transactions on Computers, vol. 52, no. 7, 2003.

- G. C. Buttazzo, Hard Real-Time Computing Systems: Predictable Scheduling Algorithms and Applications, 3rd ed., Springer, 2011.

- A. Silberschatz, P. B. Galvin and G. Gagne, Operating System Concepts, 10th ed., Wiley, 2018.

- R. D. Blumofe and C. E. Leiserson, "Scheduling Multithreaded Computations by Work Stealing," Journal of the ACM, vol. 46, no. 5, 1999.

- The Linux Kernel documentation, "Deadline Task Scheduling" (SCHED_DEADLINE).

# Appendix A. Simulator Source Code

The complete program used for all experiments. Run it with python compare_lifo_rm_edf.py.

# Comparison of LIFO, RM and EDF scheduling on periodic task sets from math import gcd, ceil from functools import reduce class Task: def __init__(self, name, burst, period, index): self.name, self.burst, self.period, self.index = name, burst, period, index class Job: def __init__(self, task, release): self.task = task self.release = release self.deadline = release + task.period self.remaining = task.burst self.first_run = None self.finish = None def label(self): return f"{self.task.name}.{self.release // self.task.period + 1}" # ---------------- policies ---------------- class Policy: preemptive = True def __init__(self): self.ready = [] def key(self, job, current): raise NotImplementedError def select(self, current): if not self.ready: return None if current is not None and not self.preemptive: return current return min(self.ready, key=lambda j: self.key(j, current)) class LIFO_NP(Policy): """Non-preemptive LIFO: when the CPU is free, the most recently released job runs.""" name = "LIFO-NP" preemptive = False def key(self, j, cur): return (-j.release, -j.task.index) class LIFO_P(Policy): """Preemptive LIFO: a newly released job immediately takes the CPU.""" name = "LIFO-P" def key(self, j, cur): return (-j.release, -j.task.index) class RM(Policy): """Rate Monotonic: shorter period = higher fixed priority.""" name = "RM" def key(self, j, cur): return (j.task.period, j.task.index) class EDF(Policy): """Earliest Deadline First: earliest absolute deadline first.""" name = "EDF" def key(self, j, cur): return (j.deadline, j is not cur, j.task.index) POLICIES = [LIFO_NP, LIFO_P, RM, EDF] # ---------------- simulator ---------------- def lcm(nums): return reduce(lambda a, b: a * b // gcd(a, b), nums) def simulate(spec, Pol, trace=False): tasks = [Task(n, c, t, i) for i, (n, c, t) in enumerate(spec)] pol = Pol() horizon = lcm([t.period for t in tasks]) timeline, jobs, missed, steps = [], [], [], [] current, preemptions = None, 0 for time in range(horizon): for j in jobs: if j.finish is None and j.deadline <= time and j not in missed: missed.append(j) pol.ready.remove(j) if j is current: current = None for t in tasks: if time % t.period == 0: j = Job(t, time) jobs.append(j) pol.ready.append(j) ready_snapshot = [j.label() for j in pol.ready] chosen = pol.select(current) if current is not None and chosen is not current: preemptions += 1 current = chosen if trace: steps.append((time, ready_snapshot, chosen.label() if chosen else "idle")) if current is None: timeline.append(None) continue if current.first_run is None: current.first_run = time timeline.append(current.task.name) current.remaining -= 1 if current.remaining == 0: current.finish = time + 1 pol.ready.remove(current) current = None for j in jobs: if j.finish is None and j.deadline <= horizon and j not in missed: missed.append(j) counted = [j for j in jobs if j.deadline <= horizon] switches = sum(1 for a, b in zip(timeline, timeline[1:]) if a and b and a != b) return dict(tasks=tasks, horizon=horizon, timeline=timeline, jobs=counted, missed=missed, preemptions=preemptions, switches=switches, steps=steps) def metrics(res): done = [j for j in res["jobs"] if j.finish is not None] resp = [j.finish - j.release for j in done] wait = [j.first_run - j.release for j in done] busy = sum(1 for x in res["timeline"] if x) n = len(res["jobs"]) return dict(jobs=n, done=len(done), missed=len(res["missed"]), miss=100 * len(res["missed"]) / n, avg_resp=sum(resp) / len(resp) if resp else 0, max_resp=max(resp) if resp else 0, avg_wait=sum(wait) / len(wait) if wait else 0, preempt=res["preemptions"], switches=res["switches"], cpu=100 * busy / res["horizon"]) def per_task(res): out = [] for t in res["tasks"]: js = [j for j in res["jobs"] if j.task is t] done = [j for j in js if j.finish is not None] resp = [j.finish - j.release for j in done] out.append(dict(task=t.name, jobs=len(js), missed=len(js) - len(done), avg=sum(resp) / len(resp) if resp else None, max=max(resp) if resp else None)) return out # ---------------- analysis ---------------- def utilization(spec): return sum(c / t for _, c, t in spec) def ll_bound(n): return n * (2 ** (1 / n) - 1) def hyperbolic(spec): p = 1 for _, c, t in spec: p *= c / t + 1 return p def rta(spec): """Response Time Analysis for RM (deadline = period).""" order = sorted(spec, key=lambda x: x[2]) rows = [] for i, (n, c, t) in enumerate(order): r = c while True: nr = c + sum(ceil(r / tj) * cj for _, cj, tj in order[:i]) if nr == r or nr > t: r = nr break r = nr rows.append((n, c, t, r, r <= t)) return rows # ---------------- scenarios ---------------- SCENARIOS = { "S1 Light load": [("T1", 1, 4), ("T2", 1, 6), ("T3", 2, 12)], "S2 Above the RM bound": [("T1", 1, 4), ("T2", 2, 6), ("T3", 3, 12)], "S3 High load": [("T1", 2, 5), ("T2", 2, 8), ("T3", 3, 10)], "S4 Harmonic full load": [("T1", 2, 4), ("T2", 2, 8), ("T3", 4, 16)], "S5 Five tasks": [("T1", 1, 5), ("T2", 1, 6), ("T3", 2, 10), ("T4", 2, 15), ("T5", 3, 30)], "S6 Overload": [("T1", 2, 4), ("T2", 3, 6), ("T3", 2, 8)], } if __name__ == "__main__": for name, spec in SCENARIOS.items(): u = utilization(spec) print("=" * 72) print(f"{name} U = {u:.3f} RM bound = {ll_bound(len(spec)):.3f}") print("=" * 72) print(f"{'Algorithm':<10}{'Jobs':<6}{'Done':<6}{'Miss':<6}{'Miss%':<8}" f"{'AvgR':<7}{'MaxR':<7}{'AvgW':<7}{'Preempt':<9}{'CPU%':<6}") for P in POLICIES: m = metrics(simulate(spec, P)) print(f"{P.name:<10}{m['jobs']:<6}{m['done']:<6}{m['missed']:<6}" f"{m['miss']:<8.1f}{m['avg_resp']:<7.2f}{m['max_resp']:<7}" f"{m['avg_wait']:<7.2f}{m['preempt']:<9}{m['cpu']:<6.1f}") print()

# Appendix B. Raw Program Output

======================================================================== S1 Light load U = 0.583 RM bound = 0.780 ======================================================================== Algorithm Jobs Done Miss Miss% AvgR MaxR AvgW Preempt CPU% LIFO-NP 6 6 0 0.0 2.00 4 0.83 0 58.3 LIFO-P 6 6 0 0.0 2.00 4 0.83 0 58.3 RM 6 6 0 0.0 1.67 4 0.50 0 58.3 EDF 6 6 0 0.0 1.67 4 0.50 0 58.3 ======================================================================== S2 Above the RM bound U = 0.833 RM bound = 0.780 ======================================================================== Algorithm Jobs Done Miss Miss% AvgR MaxR AvgW Preempt CPU% LIFO-NP 6 5 1 16.7 2.60 5 0.80 0 75.0 LIFO-P 6 5 1 16.7 2.60 6 0.60 1 75.0 RM 6 6 0 0.0 3.00 10 0.67 2 83.3 EDF 6 6 0 0.0 2.83 7 1.00 1 83.3 ======================================================================== S3 High load U = 0.950 RM bound = 0.780 ======================================================================== Algorithm Jobs Done Miss Miss% AvgR MaxR AvgW Preempt CPU% LIFO-NP 17 15 2 11.8 3.20 5 0.93 0 85.0 LIFO-P 17 14 3 17.6 3.14 5 0.43 4 82.5 RM 17 16 1 5.9 3.38 9 0.69 5 92.5 EDF 17 17 0 0.0 3.88 7 1.53 1 95.0 ======================================================================== S4 Harmonic full load U = 1.000 RM bound = 0.780 ======================================================================== Algorithm Jobs Done Miss Miss% AvgR MaxR AvgW Preempt CPU% LIFO-NP 7 6 1 14.3 3.67 8 1.33 0 87.5 LIFO-P 7 6 1 14.3 3.67 8 1.33 0 87.5 RM 7 7 0 0.0 4.57 16 1.43 1 100.0 EDF 7 7 0 0.0 4.57 16 1.43 1 100.0 ======================================================================== S5 Five tasks U = 0.800 RM bound = 0.743 ======================================================================== Algorithm Jobs Done Miss Miss% AvgR MaxR AvgW Preempt CPU% LIFO-NP 17 15 2 11.8 2.60 9 1.13 0 73.3 LIFO-P 17 15 2 11.8 2.60 9 1.13 0 73.3 RM 17 17 0 0.0 2.88 15 1.06 3 80.0 EDF 17 17 0 0.0 2.88 15 1.06 3 80.0 ======================================================================== S6 Overload U = 1.250 RM bound = 0.780 ======================================================================== Algorithm Jobs Done Miss Miss% AvgR MaxR AvgW Preempt CPU% LIFO-NP 13 9 4 30.8 3.22 5 0.78 0 95.8 LIFO-P 13 8 5 38.5 2.75 5 0.25 3 95.8 RM 13 8 5 38.5 2.75 5 0.00 5 100.0 EDF 13 8 5 38.5 3.50 8 1.38 0 100.0